ABSTRACT SYMPOSIUM PROGRAM AREA NAME: 2015 Joint Southeastern/Southwest Regional Meeting. ABSTRACT SYMPOSIUM NAME: Mobile Analysis - Oral. CONTROL ID: 2361506. TITLE: Fabrication and Characterization of Nanoscale Pillar Arrays for Planar Chromatography. PRESENTER: Michael Sepaniak. PRESENTER (E-MAIL ONLY): msepaniak@utk.edu. AUTHORS (FIRST NAME, LAST NAME): Michael J. Sepaniak1. INSTITUTIONS (ALL): 1. Dept of Chem, Univ of Tennessee, Knoxville, TN, United States.. PRESENTATION TYPE: Oral Only. ABSTRACT BODY: Abstract: This work evaluates silicon nano-pillar arrays for use in planar chromatography. Electron beam lithography (EBL) and Dewetting (DW) protocols are used to create nano thin layer chromatography platforms. The pillar heights for all systems investigated are 1-2 micron. The pillar diameter/pitch is 400/550-700 nm for the EBL arrays and 230/640 nm for the DW. A porous silicon oxide layer is added to each of the pillar arrays and then a C18 stationary phase created. Wicking flow was studied with permeability being markedly higher than for traditional planar platforms. The plate height using Sulforhodamine test dye was as low as 0.1 micron and reproducible. Separations of fluorescent dyes, environmentally significant derivatized-amines, and anti-tumor drugs are illustrated. Current efforts include adding a centrifugal force component to enhance and optimize flow rates. Since the pillar platforms are diminutive and driven by wicking flow they are suitable for mobile, in field use.. DECISION STATUS:. ABSTRACT STATUS: submitted Withdraw.